Site Safety Inspection

(Construction Only)

Note:  This check sheet is intended to assist inspections of construction sites.  A list such as this cannot be comprehensive and initiative should be used when inspecting a site. Many of the questions can only be answered by

talking to site staff and workforce.  Amend this form to suit your own personal use.

| Site: |  |  | Reference No. |
| --- | --- | --- | --- |
| Site Manager: |  |  |  |
| Inspected by: |  |  | Date |
| Next Inspection due: |  |  |  |

| No. | Activity | Assessed |  | Comments, opportunities for improvement |
| --- | --- | --- | --- | --- |
|  |  | Yes | No |  |
| 1. | Management of Health & Safety |  |  |  |
| 1.1 | Were you made to sign in and out/ have you signed in? |  |  |  |
| 1.2 | Are operatives made to sign in and out? Are they doing so? |  |  |  |
| 1.3 | Are the CDM Site files available on site? Is the CDM board displayed? |  |  |  |
| 1.4 | Are the method statements available for all contractors and their activities and is there evidence that the workforce have been briefed and understand them? |  |  |  |
| 1.5 | Is it clear to visitors where they are supposed to report? Are visitors booked in? Is there a record of CSCS card? Is there an induction for operatives and visitors? Are records available on site for inspection? |  |  |  |
| 1.6 | Is appropriate PPE being worn by all site personnel? Eye protection, hearing protection, HI-VIZ, hard hats, boots, gloves etc |  |  |  |
| 1.7 | Are the F10 notification, Employers liability insurance, H+S policy statement, HSE law poster, COSHH poster, site rules, fire plan clearly displayed? |  |  |  |
| 1.8 | Is there adequate safety signage at the entrance and also around the site? |  |  |  |
| 1.9 | Is there evidence of the competence of the workforce? Copies of employees training certification on file? |  |  |  |
| 1.10 | Are fire extinguishers & First Aid facilities available? Qualified First Aid personnel on site? How many? Is the hospital route displayed and emergency contact numbers available? First aid box adequately stocked? Accident book on site compliant with current legislation especially data protection? First aid signage? Eye wash. Preferably single vials to prevent cross contamination. |  |  |  |
| 1.11 | Adequately lit escape route to include emergency lights. |  |  |  |
| 1.12 | Hot work permits |  |  |  |
| 1.13 | Storage of LPGs and hazardous materials? |  |  |  |
| 1.14 | Are there systems on site for monitoring weather conditions and changes which could pose a threat? |  |  |  |
| 1.15 | Inspection register for scaffold, plant, excavations etc. |  |  |  |
| 1.16 | Noise/Vibration/Asbestos considered? |  |  |  |
| 2. | General Public |  |  |  |
| 2.1 | Is the site enclosed, or is there other protection for the public? |  |  |  |
| 2.2 | Are there any areas where trespassers might gain access to site? How will you stop access? |  |  |  |
| 2.3 | Are there any trips or similar hazards in public areas? |  |  |  |
| 2.4 | Is there adequate signage for the public? Warnings of danger, keep out, contact numbers for out of hours Emergency response? |  |  |  |
| 3. | Slips and Trips |  |  |  |
| 3.1 | Are there clear designated pedestrian walkways? Areas for material storage? |  |  |  |
| 3.2 | Is the site accessible, kept tidy with no materials or debris causing trip/slip hazards? |  |  |  |
| 4. | Site Traffic |  |  |  |
| 4.1 | Is there a traffic management plan in place, updated as site progresses? Have all personnel seen it? |  |  |  |
| 4.2 | Does the plan identify separate pedestrian & vehicle routes where possible? |  |  |  |
| 4.3 | Are obstacles such as excavations and scaffolds clearly marked or protected? |  |  |  |
| 4.4 | Does any plant need a banksman and if so, is there one present? |  |  |  |
| 4.5 | Are there any potentially dangerous slopes where machinery might slide or overturn? |  |  |  |
| 4.6 | Are operators of dumpers wearing lap-belts? Are flashing beacons and sounders working on all plant? ROPS or FOPS fitted and in correct use Is there a speed limit on site? Is it obeyed? |  |  |  |
| 4.7 | Deliveries and waste removal? |  |  |  |
| 5. | Falls from Height |  |  |  |
| 5.1 | Has a safe system of work been put in place to ensure fall protection or prevention, including excavations? |  |  |  |
| 5.2 | Are all open edges and holes appropriately protected to guard against falls of people and materials? |  |  |  |
| 5.3 | Have all scaffolds been erected appropriately to meet the needs for which they are constructed? Are recorded inspections/handover tickets available? |  |  |  |
| 5.4 | Are all working platforms fully & properly boarded out? Boards in good condition? |  |  |  |
| 5.5 | Are guardrails, toe boards and brick guards been fitted? |  |  |  |
| 5.6 | Stored materials are evenly distributed on the scaffold platforms and not excessive or above guardrails? |  |  |  |
| 5.7 | Are ladders used only as means of access, except for short duration work? |  |  |  |
| 5.8 | Are ladders secured before use to prevent slipping? Ladders correct length? |  |  |  |
| 6. | Lifting Equipment |  |  |  |
| 6.1 | Is any lifting equipment suitable for this purpose? Are weekly checks undertaken? |  |  |  |
| 6.2 | Is the load secure? |  |  |  |
| 6.3 | Is the lifting equipment working on solid even ground. |  |  |  |
| 6.4 | Is there a certification of thorough examination for the crane and the lifting accessories? |  |  |  |
| 6.5 | Is there a suitable Lift Plan and an appointed person? Operators license on file? |  |  |  |
| 6.6 | Is there a Safe Working Load to comply with? |  |  |  |
| 7. | Services |  |  |  |
| 7.1 | Are appropriate precautions in place to safe guard against striking underground services during the work? |  |  |  |
| 7.2 | Has identification of buried services or services exposed during the works been carried out & recorded |  |  |  |
| 7.3 | Have all overhead lines been identified and steps taken to remove divert or mark lines to prevent contact? |  |  |  |
| 7.4 | Is there a permit to dig system in place? |  |  |  |
| 7.5 | Are goal posts and signage being used to warn of overhead services? |  |  |  |
| 7.6 | Temporary electric certificate |  |  |  |
| 7.7 | PAT records and temporary supply certificates? Festoon bulbs protected and terminals sealed? 110 V or battery tools only |  |  |  |
| 8. | Confined Spaces |  |  |  |
| 8.1 | Are there any designated confined spaces? |  |  |  |
| 8.2 | If so is there a thorough risk assessment and method statement and are the requirements being adhered to? Does it include an adequate rescue plan? |  |  |  |
| 8.3 | Are there any other locations which might be a confined space but which have not been designated? |  |  |  |
| 9. | Manual Handling |  |  |  |
| 9.1 | Have positive steps been taken to identify materials likely to be supplied in bulk, in unacceptably large size units? (blocks, dry goods, cement) |  |  |  |
| 9.2 | Are workers manually lifting any large items of equipment or material? What is the largest? |  |  |  |
| 9.3 | Is COSHH information available for materials being used on site? |  |  |  |
| 10. | Welfare |  |  |  |
| 10.1 | Are all welfare facilities reasonably accessible to all workers on site and clean? Drying rooms? |  |  |  |
| 10.2 | Is there accommodation available for sitting, heating water & preparing food, potable water? |  |  |  |
| 10.3 | Are there adequate numbers of toilets, & wash basins with warm water, cleaners & towels and are they being well maintained? |  |  |  |
| 10.4 | If cooking on site, do they hold a current food hygiene certificate? |  |  |  |
| 10.5 | Are there fire extinguishers available in the welfare areas? |  |  |  |
| 11. | Additional Comments |  |  |  |