School Advice Note

Glazing

- The Schools glazing must comply with the Workplace (Health, Safety and Welfare) Regulations 1992.  The potential risk areas are as follows:

  - Glazed doors and door side panels that may be mistaken for doors;

  - Low level glazing which is within 800mm of floor ground level;

  - Glazing between 800mm and 1500mm of floor or ground level where there is a special risk of personal damage.  Examples of this could be shower rooms, playgrounds, hallways etc;

  - Balustrades (rail around landings);

  - High level glass adjacent to playgrounds, sports fields etc;

- If glazing in these areas does present a risk, there are four possible options that can be taken:

  - The glass is replaced with different material (i.e. insulating panel or plywood);

  - Some kind of barrier is fixed to prevent contact with the glass surface.  This could be fixed rails, planting, and furniture;

  - The glass is replaced with safety glass;

  - A plastic film is applied to the existing glass, with anti shatter characteristics to BS6206;

- Safety films that can be used in this process have a life span in the region of 10-15 years.

- Further advice can be found in the following publication:

  - Southampton City Council Safe Working Procedure – Workplace Health, Safety and Welfare.