ASB 2 Form – Addition to Medical Record in Respect of Previous Exposure to Asbestos

This information relates only to exposure whilst being employed by previous employers.

Not exposure when employed by Southampton City Council.

See sections 2.58, 2.64 or 2.94 of the Council Policy Control of Asbestos for more information.

| - Personal Details [Please complete in CAPITALS] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Surname |  |  |  |  |  |  | Forename(s) |  |  |  |  |  |  |  |  | Occupation |  |  |  |
| Hub/Directorate |  |  |  |  |  |  | Team |  |  |  |  |  |  |  |  | Pay No |  |  |  |
| - Details Of Possible Exposure To Asbestos (to be completed by employee – please give as much information as possible. If you do not know details such as the type, class, level in air, enter U/K but please describe the work as carefully as possible) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date | Employers Name and address |  |  | Type |  |  |  | Class |  |  |  |  | Work Being Carried Out | Duration Of Exposure |  |  | Estimate Of Level In Air* |  | Comment |
|  |  |  |  | W | Br | Bi |  | INS | AIB | AC | Other |  |  |  |  |  |  |  |  |
| W = White INS = Asbestos Insulation or Coating * When estimating the level in air the table on the reverse of Br = Brown AIB = Asbestos Insulation Board this form may be useful. Bi = Blue AC = Asbestos Cement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Signed: |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |

Note:

Line Manager to send copies to Occupational Health and keep a copy locally.

Typical fibre concentrations for work with Asbestos lagging, Asbestos coating and Asbestos insulation board

| Technique | Typical Exposures (f/ml) | Comment |
| --- | --- | --- |
| Controlled wet stripping of lagging and sprayed coatings | Up to 1 | Thorough soaking of the lagging with a wetting agent followed by careful removal |
| Uncontrolled dry stripping of lagging | 1 – 100 | Stripping dry or where dry patches are encountered |
| Uncontrolled dry stripping of sprayed coatings | Up to 3 | Unscrewing (with shadow vacuuming) with the spray application of a wetting agent on unsealed surfaces |
| Breaking and ripping out AIB | 5 – 20 | Carried out dry with no unscrewing |
| Drilling AIB overhead | 5 – 10 | No LEV |
| Drilling vertical columns | 2 – 5 | No LEV |
| Use of jig saw on AIR | 5 – 20 | No LEV |
| Hand sawing | 5 – 10 | No LEV |

Typical fibre concentrations for work with asbestos cement

| Activity |  | Typical exposure (f/ml) |
| --- | --- | --- |
| Machine sawing with exhaust ventilation |  | Up to 2 |
| Machine cutting without exhaust ventilation | Abrasive disc cutting | 15 - 25 |
|  | Circular saw | 10 - 20 |
|  | Jig saw | 2 – 10 |
| Hand sawing |  | Up to 1 |
| Machine drilling |  | Up to 1 |
| Removal of asbestos cement sheeting |  | Up to 0.5 |
| Stacking of asbestos cement sheets |  | Up to 0.5 |
| Remote demolition of asbestos cement structures dry* |  | Up to 0.1 |
| Remote demolition of asbestos cement structures wet* |  | Up to 0.01 |
| Cleaning asbestos cement | Roofing | Vertical Cladding |
| Dry brushing (wire) | 3 | 5 - 8 |
| Wet brushing (wire) | 1 – 3 | 1 – 2 |