School Advice Note

Toilet and Washing Facilities for Pupils

- The only specific legislation on toilets and washing facilities for pupils in England is contained in The School Premises (England) Regulations 2012 for Maintained and The Education (Independent School Standards) Regulations 2014 for Academies, Trust and Voluntary Aided schools. This Education legislation specifies the number of toilets and washbasins that pupils must have. There is no mention in the legislation of cleanliness and condition.

- This legislation sets out the number of toilets and washbasins that should be provided according to the number of pupils in all existing and new maintained, Voluntary Controlled schools and Academies, Trust and Voluntary Aided schools.

General planning

- Toilet facilities need to be planned and designed so that:

  - Hand washing facilities are provided within or in the immediate vicinity of every toilet;

  - The rooms containing them are adequately ventilated and lit;

  - They are located in areas around the school that provide easy access for pupils and allow for informal supervision by staff, without compromising pupils’ privacy.

  - With the unisex provision, the privacy of the occupant needs to be ensured and this will be achieved by, for example, having a full height door.

Numbers of Toilets and washbasins

- The Regulation does not set the minimum number of fittings to be provided in relation to the ages and numbers of pupils.

- Generally the needs of younger pupils are likely to be greater than those for older ones. So, for example, a provision of one toilet and washbasin for every ten pupils under 5 years old would be adequate, while that ratio could be doubled for pupils aged 5 -11 to one toilet and washbasin for every 20 pupils.

- For pupils over 11 one toilet per 20 pupils would be sufficient, but there is scope to reduce the number of washbasins where the washing facilities are shared.

- Separate toilet facilities for boys and girls aged 8 years or over must be provided except where the toilet facility is provided in a room that can be secured from the inside and that is intended for use by one pupil at a time.

Facilities for disabled pupils

- Each toilet for disabled pupils needs to contain one toilet and one washbasin (and possibly a shower or other wash down fitting) and have a door opening directly onto a circulation space that is not a staircase, which can be secured from the inside.

- Where possible, the number and location of accessible toilets will be sufficient to ensure a reasonable travel distance for users that does not involve changing floor levels.

- References

  - The School Premises (England) Regulations 2012

  - Standards for School Premises. Departmental advice for Local Authorities, School Leaders and Governing Bodies Supplementary guidance: Standards for school premises

  - The Education (Independent School Standards) Regulations 2014: Part 5 premises of and accommodation at schools